Line-by-line explanation (code → meaning)

import numpy as np

import matplotlib.pyplot as plt

- Import numpy for numeric/vector operations and matplotlib.pyplot for plotting simulation results.

Triangular membership function

def trimf(x, a, b, c):

if x <= a or x >= c:

return 0.0

elif x == b:

return 1.0

elif x < b:

return (x - a) / (b - a)

else:

return (c - x) / (c - b)

- trimf implements a triangular membership function with parameters (a, b, c):

  - a = left foot (membership 0 at or below a),

  - b = peak (membership 1 at b),

  - c = right foot (membership 0 at or above c).

- For x between a and b the membership rises linearly; between b and c it falls linearly.

- This returns a membership degree in [0,1].

Fuzzy input sets for error e

def err_neg(x): return trimf(x, -100, -100, 0)     # Arm far below target

def err_zero(x): return trimf(x, -20, 0, 20)       # Arm near target

def err_pos(x): return trimf(x, 0, 100, 100)       # Arm far above target

- Three fuzzy sets for the error (target − current):

  - err_neg: negative (arm well below desired). Using a=b=-100 makes the left slope vertical; effectively membership 1 near very negative and 0 at 0.

  - err_zero: centered at 0 with support [-20,20].

  - err_pos: positive (arm well above desired), symmetric to err_neg.

Fuzzy input sets for derivative of error de

def derr_neg(x): return trimf(x, -50, -50, 0)      # Error decreasing fast

def derr_zero(x): return trimf(x, -10, 0, 10)      # Error stable

def derr_pos(x): return trimf(x, 0, 50, 50)        # Error increasing fast

- Three fuzzy sets for de = d(e)/dt (rate of error change):

  - Negative (fast decreasing), zero (stable), positive (increasing).

Output fuzzy sets (control torque u)

u_universe = np.linspace(-20, 20, 401)             # Output range (torque levels)

def u_sn(x): return trimf(x, -20, -20, -10)        # Strong negative torque

def u_wn(x): return trimf(x, -15, -7, 0)           # Weak negative torque

def u_z(x):  return trimf(x, -3, 0, 3)             # Zero torque

def u_wp(x): return trimf(x, 0, 7, 15)             # Weak positive torque

def u_sp(x): return trimf(x, 10, 20, 20)           # Strong positive torque

- u_universe is the discrete set of output values used for aggregation and defuzzification (from -20 to 20).

- Five triangular output membership functions for control torque: strong neg, weak neg, zero, weak pos, strong pos.

Rule base (Mamdani inference)

def evaluate_rules(e_val, de_val):

e_neg = err_neg(e_val); e_zero = err_zero(e_val); e_pos = err_pos(e_val)

de_neg = derr_neg(de_val); de_zero = derr_zero(de_val); de_pos = derr_pos(de_val)

fired = []

# Rule table (9 total)

fired.append((min(e_neg, de_neg), u_sn))  # if e is neg and de is neg → strong neg torque

fired.append((min(e_neg, de_zero), u_wn))

fired.append((min(e_neg, de_pos), u_z))

fired.append((min(e_zero, de_neg), u_wn))

fired.append((min(e_zero, de_zero), u_z))

fired.append((min(e_zero, de_pos), u_wp))

fired.append((min(e_pos, de_neg), u_z))

fired.append((min(e_pos, de_zero), u_wp))

fired.append((min(e_pos, de_pos), u_sp))

return fired

- This is a 3×3 Mamdani rule table (9 rules). For each rule:

  - Evaluate antecedent truth degree (here min of e membership and de membership).

  - Pair (firing_strength, consequent_mf) is returned.

- Example: if error is negative and error derivative is negative (still decreasing), fire u_sn with degree min(e_neg, de_neg).

Aggregation of consequent fuzzy sets

def aggregate(fired, u_univ):

agg = np.zeros_like(u_univ)

for strength, mf_func in fired:

if strength <= 0:

continue

vals = np.array([min(strength, mf_func(u)) for u in u_univ])

agg = np.maximum(agg, vals)

return agg

- For each fired rule:

  - Clip (truncate) the rule’s consequent membership function at the rule’s firing strength: min(strength, mf(u)).

  - Aggregate (take maximum) across all clipped consequents to form the combined output fuzzy set agg(u) (the Mamdani aggregation step).

Defuzzification (centroid method)

def defuzz_centroid(combined_mf, u_univ):

num = np.sum(combined_mf * u_univ)

den = np.sum(combined_mf)

if den == 0:

return 0.0

return num / den

- Computes the centroid (center of gravity) of the aggregated output membership:

  - u* = (∑ μ(u) * u) / (∑ μ(u)).

- Returns a crisp control torque u_out.

Simulation parameters and loop

target_angle = 90.0       # degrees (desired position)

cur_angle = 0.0            # starting angle

dt = 0.1                   # time step (s)

gain = 1.0                 # effect of torque on angle

max_steps = 200

control_sat = 20.0         # torque limit

- Set target, initial angle, timestep, actuator gain, max simulation steps, and torque saturation.

time_hist, angle_hist, control_hist = [], [], []

prev_error = target_angle - cur_angle

- Arrays to record simulation history and initial previous error.

for step in range(max_steps):

e = target_angle - cur_angle

de = (e - prev_error) / dt

# Fuzzy inference

fired = evaluate_rules(e, de)

agg = aggregate(fired, u_universe)

u_out = defuzz_centroid(agg, u_universe)

# Saturation

u_out = max(min(u_out, control_sat), -control_sat)

# Update angle (simple model)

cur_angle += gain * u_out * dt

time_hist.append(step * dt)

angle_hist.append(cur_angle)

control_hist.append(u_out)

prev_error = e

# Stop if close to target

if abs(e) < 0.5 and abs(de) < 0.2:

print(f"✅ Converged at step {step}, angle = {cur_angle:.2f}")

break

- Each simulation step:

  - Compute current error e and its derivative de.

  - Fuzzify (evaluate_rules uses membership functions).

  - Perform inference (rule firing), aggregation, and defuzzification → get u_out.

  - Enforce actuator saturation.

  - Apply simple plant model: cur_angle += gain * u_out * dt (a first-order integrator-like update).

  - Record histories.

  - Terminate early if error and derivative small (convergence).

Plotting results

plt.figure(figsize=(10,4))

plt.subplot(1,2,1)

plt.plot(time_hist, angle_hist, label='Angle')

plt.axhline(target_angle, color='r', linestyle='--', label='Target')

...

plt.show()

- Two plots: angle vs time (shows tracking to target) and control (torque) vs time (shows control action).

Theory for viva — complete and concise

1. What is a fuzzy controller?

- A fuzzy controller maps inputs (linguistic/continuous) to outputs using fuzzy logic: fuzzification → rule inference → aggregation → defuzzification.

- Mamdani controller (your code) uses fuzzy sets as consequents and an output aggregation followed by centroid defuzzification.

- Common use: systems where a model is hard to derive but expert rules exist (e.g., human-style rules).

2. Key stages (with math)

- Fuzzification
Convert crisp inputs to fuzzy degrees:
μA(x)∈[0,1].
Example: e mapped to {err_neg, err_zero, err_pos} via triangular MFs.

- Rule evaluation (implication)
For a rule: IF e is E AND de is DE THEN u is U, compute firing strength:

  - Using AND = min here: α=min⁡(μE(e),μDE(de)).

- Rule implication to consequent (clipping)
For Mamdani, consequent MF is clipped by α:

  - μUclipped(u)=min⁡(α,μU(u)).

- Aggregation (combine consequents)
Combine all clipped consequents by max:

  - μagg(u)=max⁡iμUi,clipped(u).

- Defuzzification (centroid)
Crisp control:

u*=∫u⋅μagg(u) du∫μagg(u) du

(discretized as sums in code).

3. Why Mamdani?

- Intuitive, easy to encode expert rules.

- Produces smooth control action after defuzzification.

- Downside: expensive defuzzification; not ideal for high-speed control where Sugeno (with polynomial consequents) can be faster.

4. Membership functions

- Triangular MFs used here are simple and computationally cheap.

- Other MFs: trapezoidal, Gaussian, bell-shaped. Choice affects smoothness and robustness.

5. Rule design and interpretation

- Rules reflect control intuition: e.g., if error positive and increasing → apply positive torque.

- Rule base must be consistent and cover expected operating range.

6. Defuzzification methods (comparison)

- Centroid (center of gravity) — smooth, widely used.

- Mean of maxima — average of peak positions.

- Bisector — splits area in half.

- Weighted average (used in Sugeno) — computationally cheaper if consequents are crisp functions.

7. Relation to classical PID

- Fuzzy PD/PI controllers mimic PID behavior: error ~ proportional, derivative ~ derivative action.

- Fuzzy controllers can handle nonlinearities and saturations better using heuristic rules.

8. Stability & tuning

- Fuzzy controllers do not have straightforward analytical stability proofs like linear PID, but:

  - Use simulations, Lyapunov-based methods, or design frameworks (fuzzy model-based control) for guarantees.

- Tune by:

  - Adjusting MF shapes and ranges.

  - Changing rule consequents and scaling factors (input/output gains).

  - Modifying defuzzification resolution or aggregation strategy.

9. Practical points in your simulation

- Scaling: Inputs used in membership functions must be in same units/scale used to design MFs. Here err domain assumes degrees and large negative/positive bounds.

- Saturation: Implemented to represent actuator limits.

- Plant model: Very simple (angle += gain * torque * dt); realistic systems need dynamics (inertia, friction), often second-order differential equations.

- Discrete universe for defuzzification: u_universe discretizes the output domain for numeric centroid; resolution affects accuracy.

Common viva questions & model answers

Q: Why use fuzzy control over PID?
A: Fuzzy control handles nonlinearities, uncertainty, and expert heuristic rules without requiring an exact plant model. It can be more robust in poorly modeled systems. However, PID has analytic tuning and stability properties.

Q: Why choose triangular MFs?
A: Triangular MFs are simple, cheap to compute, and easy to interpret. They work well when precise shapes are not critical; can be replaced by smoother MFs if needed.

Q: What is the firing strength and how is it computed?
A: Firing strength = degree to which antecedent is true. For multi-input antecedent with AND, firing strength = min of individual membership degrees.

Q: Why use min for AND and max for OR?
A: min and max are standard t-norm and t-conorm. They satisfy desirable properties (commutative, associative, monotonic). Other logical operators exist (product, Lukasiewicz), but min/max are most common in Mamdani.

Q: Explain defuzzification centroid and its tradeoffs.
A: Centroid computes center of gravity of aggregated MF; yields smooth output but requires numerical integration (computational cost). It’s intuitive and widely used for smooth control.

Q: How do you tune the fuzzy controller?
A: Tune MF ranges and shapes, adjust scaling (input/output gains), and modify rule consequents. Use simulations, optimization (GA/PSO) or expert knowledge.

Q: How would you make controller faster or more reliable?
A: Use Sugeno-type controller (crisp functions) for faster defuzzification, implement MF adaptive tuning, or combine with model-based control. Also model plant more accurately and include integrator/damping.

Practical tips to say in viva (short bullets)

- Emphasize fuzzification → inference → aggregation → defuzzification flow.

- Mention choice of Mamdani and centroid and why (intuitive, smooth).

- Point out need for feature scaling and realistic plant dynamics.

- Note that rule base encodes human expertise and that fuzzy controllers are good where no accurate mathematical model is available.